SKM cameras daemon input protocol

| Input text | Meaning |
| --- | --- |
| Unknown objID=xx | Set ‘Unknown’ state |
| Startup objID=xx | Client (Retriever) is starting up. About to load configuration of the camera. |
| Connecting objID=xx | Trying to connect to device |
| Streaming pos=yy, objID=xx | Streaming from device go OK at posintion yy |
| Stream_lost objID=xx, code=zz | Streaming stopped. Stop reason code is zz (0-unknown, 1-device closed socket, 2-network failure, ...) |
| Axis camera video loss¦restore reports: |  |
| Videoloss_objID=xx | Report from Axis video loss listener. |
| Videorestored_objID=xx | Report from Axis video loss listener. |

| Retriever state | State stored in OM |
| --- | --- |
| Unknown objID=xx | Unknown |
| Startup objID=xx | Unknown |
| Connecting objID=xx | Unknown |
| Streaming pos=N, objID=xx | Camera ONline |
| Stream_lost objID=xx | Camera OFFline |

How Retriever’s state stored in OM

INPUT Serial port protocol from CC&C side

| Command (Advantor side) | Response (SKM) | description |
| --- | --- | --- |
| LCM_X_Y<CR> | LCM_X_Y<CR><LF>TC8800_>_ | Put camera id=X to monitor Y, <CR>=0xD, <LF>=0xA, _ is space (0x20) |
| prepos_X_Y<CR> | prepos_X_Y<CR><LF>TC8800_>_ | Move PTZ camera id=X to a preset position Y |
| video-status<CR> | video-status[0x15][..N bytes..][X]<CR><LF>TC8800_>_ | cameras' status. where [X] is checksum, N bytes is number of reported cameras divided by 4, each byte contains info for 4 cameras: 2 bits per camera - left bit is 1 when video loss monitoring enabled, right bit - video present (1-yes/0-no) |
| ver<CR> | ver<CR><LF>TC8800_>_ | ping |
| INSTALL-CAM_X_Y<CR> | INSTALL-CAM_X_Y<CR><LF>TC8800_>_ | set video loss detection for camera X: ON when Y=1, OFF when Y=0 |
| UNINSTALL-ALL<CR> | UNINSTALL-ALL<CR><LF>TC8800_>_ | set video loss detection off on all cameras |

- ID of a camera (for Serial protocol ONLY) decreased by 100. so camera with SKM OBJID=101 Advantor will know as ID=1, 102->2 etc.

- ‘video-status’ report also send to Advanor side when video-loss|restore detected on subscribed camera.

- Svn...advantor/doc/mail.txt : The Burle response is to return each command along with "<LF>TC8800> " where <LF> is an 0x0A.

- For ERROR case response has next structure: <command><0xD><0xA>ERROR:  argument(s) out of range<0D><0A><0D><0A>TC8800 >