Sightlogix / Autoslew installation and testing instructions

- Source code overview

The source code is broken apart into two separate parts: autoslew & Sightlogix server.  The autoslew code was originally written for the Athena integration with Raytheon in 2007.  It will accept a geo coordinate of an object and given the camera location automatically position the camera to be in proper position to view that object.  Currently, it works with pan / tilt but will not zoom at all.  The Sightlogix server code was written to interact with the sightlogix camera to retrieve lat / lon coordinates and send them to the autoslew code.

The components work by sending messages to each other over the message bus. The XML messaging between the two components looks like this:

Message from Sightlogix => Autoslew

<PTZ CMD="AutoSlew" CASE="DirectGeoCoordinates">

<TARGET LAT="xxx" LONG="xxx" ELEV="xxx" SIZE="xx,yy"/>

<DETECTOR LAT="xxx" LONG="xxx" ELEV="xxx" DEFLECTION="xx"/>

<AUTO_SLEW METHOD="[ALL|AUTOSELECT]"  TARGET_FOV="50%">

<CAM OBJID="xxx1"/>

<CAM OBJID="xxx1"/>

<CAM OBJID="xxx1"/>

</AUTO_SLEW>

</PTZ>

Message from Autoslew => PTZ code

<PTZ_Command>do.ptz?dev=11&mode=abs&PT=xx,yy</PTZ_Command>

- Autoslew can be found in the repository under: Analytics/src/autoslew

- Sightlogix can be found in the repository under: Analyatics/src/SightlogixServer

- Building

The components all assume mbus, imp, and analytics have been checked out of the repository.  Generally speaking, the external dependencies are libcurl, xerces, and message bus.  All makefiles use relative path so there is no reason to modify them unless the baseline structure has been changed.

To build autoslew:

% cd analytics/src/autoslew

% make

To build Sightlogix:

% cd analytics/src/SightlogixServer

% make

All executables are placed in analytics/bin.

- Setup and Execution

The system assumes that $APL_CONF/<devid>/conf has been set up to include SLEW_FROM_OBJIDS, CAM_GEO_LAT, CAM_GEO_LONG, CAM_GEO_ALT, and CAM_GEO_NORTH_OFFSET.  SLEW_FROM_OBJIDS is necessary for FLIR cameras, and should be a comma separated list of object IDs of Sightlogix cameras.  Cameras to be slewed must include CAM_GEO_NORTH_OFFSET, CAM_GEO_LAT, CAM_GEO_LONG, and CAM_GEO_ALT.

Examples can be seem below:

/opt/sarch/var/conf/1/conf:

SLEW_FROM_OBJIDS=104,105

/opt/sarch/var/conf/5/conf:

CAM_GEO_NORTH_OFFSET=10

CAM_GEO_LAT=10.0

CAM_GEO_LON=-15

CAM_GEO_ALT=10.0

To simplify the process of running, a script has been created under analytics/bin called “runSightlogix.sh”.  This file also includes documentation on how to run the components. There are 2 executables: repositionClient & SightlogixServer.  The starting order is unimportant.  SightlogixServer takes one command line argument, which is the devid of the FLIR camera.  RepositionClient takes no arguments.   LD_LIBRARY_PATH must include imp/lib and mbus/lib.

#!/bin/bash

### TO CONFIGURE THIS TO YOUR LOCAL ENVIRONMENT.

### 1. Check LD_LIBRARY_PATH to ensure that it is correct.

export LD_LIBRARY_PATH=../../imp/lib:../../mbus/lib

## 2. Check to see if the Sightlogix ID points to correct ID to the FLIR you wish to slew based on sightlogix tracking

SIGHTLOGIX_ID=1

./repositionClient &

./SightlogixServer $SIGHTLOGIX_ID &

As SightlogixServer runs, it will print some debugging information which may be useful, sample output is below:

[apl@HM1 bin]$ ./SightlogixServer 1

Polling 207.207.160.242

....*.*.*.

In the output, every time a message is received from Sightlogix, a ‘.’ is printed.  If the message contains an object, a ‘*’ is also printed.  This is a compact but useful way to check the communications between the Sightlogix camera and the system.  If you wish to look at the XML output from the camera in more detail, the following command can also be used:

%  curl --digest -u sightlogix:push2edg "http://207.207.160.242:80/cgi/metadata.cgi?template=xml&ObjectStatus=2"

<?xml version="1.0"?>

<!--Data developed by SightLogix (http://www.SightLogix.com)-->

<Camera xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xsi:noNamespaceSchemaLocation="http://www.sightlogix.com/SightLogixMetadata.xsd">

<CameraParameters SerialNumber="000000217">

<CameraName>SL_camera</CameraName>

</CameraParameters>

<CameraStatus Time="948404899820212">

<FOV>46.5</FOV>

<Bearing>

<Pan>-140.4</Pan>

<Tilt>-10.3</Tilt>

</Bearing>

<Position>

<Lat>38.91163</Lat>

<Long>-77.47041</Long>

<Elev>10.00</Elev>

</Position>

<Alarm>false</Alarm>

<ObjectCount>0</ObjectCount>

<AlarmedObjectCount>0</AlarmedObjectCount>

</CameraStatus>

See the Sightlogix documentation for information on the XML & the parameters used in the CURL command.

IMPORTANT:

When debugging this, please note that atof gives about 7 digits worth of precision.  Anything past that may not be reliable.  Be careful how you print this out – printf(“%g”, lat) for example, will round to 4 digits but the original data is still OK!