# VideoNEXT HTTP PTZ protocol

To activate PTZ command your application should request URL:

http://Stratus-server/ptz/cgi-bin/send_message.pl?data=%3CPTZ_Command%3Edo.ptz?dev=101%26mode=speed%26pt=0,0%26%3C/PTZ_Command%3E

Where:

X – Device ID (number). It is the same as OBJID.

### Step mode

Move camera step by step

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=step | move | up, down, left, right, upleft, upright, downright, downleft | do one step in specific direction |
|  | zoom | in, out |  |
|  | focus | near, far |  |
|  | iris | open, close |  |
|  | gain | up, down |  |

Example:

dev=3&mode=step&move=right

dev=1&mode=step&zoom=out

### Smooth mode

Move camera smoothly until stop (or any other command) is sent

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=smooth | move | stop,up, down, left, right, upleft, upright, downright, downleft | starts moving in specific direction |
|  | zoom | in, out, stop |  |
|  | focus | near, far, stop |  |
|  | iris | open, close, stop |  |
|  | gain | up, down, stop |  |

Example:

dev=3&mode=smooth&move=right

dev=3&mode=smooth&move=stop

### Presets

Working with preset positions. Preset positions are stored in cameras (Sony, Pelco)

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=preset | goto | 1..6 | goto preset position |
|  | save | 1..6 | save current position |
|  | clear | 1..6 | delete preset position |

Example:

dev=3&mode=preset&goto=2

### Speed mode

Move (or change zoom, focus, iris etc) camera with specific speed

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=speed | pt | -100..100,-100..100 | move with speeds for pan and tilt. To stop moving you should send command mode=speed&pt=0,0 |
|  | z | -100..100 | zoom in/out with speed. to stop zoom you should send command mode=speed&z=0 |
|  | focus | -100..100 |  |
|  | irsis | -100..100 |  |
|  | gain | -100..100 |  |

Example:

dev=3&mode=speed&pt=50,-35 (move right and down with speeds 50 and 35)

dev=3&mode=speed&pt=0,0 (stop)

dev=3&mode=speed&z=20 (zoom in with speed 20)

### Absolute Positioning

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=abs | pt | Pdegree,Tdegree | degrees: Pdegree > 0 – right, Pdegree < 0 – left, Tdegree > 0 – down, Tdegree < 0 – up |
|  | z | 0..100 |  |
|  | center | 1 | centers camera |

Example:

dev=3&mode=abs&pt=50,135

### Hardware reset / set defaults

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=hardware | do | defaults, reset | defaults – set all parameters to default values; reset – hardware reset of the camera |

Example:

dev=2&mode=hardware&do=reset

### Relative positioning (center)

This mode can be used when you centering video by clicking on it. You MUST specify both xy and size parameters to do that.

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=rel | xy | x,y | Center camera to x,y position (based on video resolution) |
|  | size | WIDTHxHEIGHT | Video resolution |

Example:

dev=2&mode=rel&xy=100,120&size=320x240 - move camera, so whatever was on 100x120 on video will be in the center (160x120).

### Settings mode

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=settings | whitebalance | auto, indoor, outdoor, off, onepushwb, atw, manual, onepushtrigger |  |
|  | gain | auto, reset, up, down, direct |  |
|  | autolevel | on, off |  |
|  | shutter | auto, manual |  |
|  | polarity | white, black |  |
|  | autofocus | on, off, auto |  |
|  | autoiris | on, off, auto |  |
|  | digitalzoom | on, off |  |

Example:

dev=1&mode=settings&autofocus=off

dev=1&mode=settings&gain=direct&pq=XY

### Sequence mode

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=sequence | save | 1..99 | Save sequence |
|  | load | 1..99 | Load sequnce |
|  | points | preset1:secs {,presentN:secs} | Sequence of presets to tour |
|  | speed | 1..99 | Speed for camera to go between presets |

Example:

dev=1&mode=sequnce&points=1:7,2:9&speed=10&save=12

dev=1&mode=sequnce&load=92

### Lock mode

| Mode | Commands | Values | Description |
| --- | --- | --- | --- |
| mode=lock | cmd | info | Return lock info |
|  |  | lock | Try to obtain lock. Return error if device is locked by another user |
|  |  | override | Try to override existing lock |
|  |  | release | Release lock |

Example:

dev=1&mode=lock&cmd=override

dev=1&mode=lock&cmd=release

# GEO-PTZ commands

To send target coordinates with GEO-PTZ command your application should request URL

http://Stratus-server/ptz/cgi-bin/send_message.pl?data=%3CPTZ_Command%3Edo.ptz?target=t123%26mode=direct%26lat=38.91145%26long=-77.47004%26elev=5%26fov=50%26slew=101,102%26%3C/PTZ_Command%3E

Where:

t123 – Target ID (text)

### Direct GEO coordinates

Provide target location using geographic coordinates

| Mode | Parameter | Values | Description |
| --- | --- | --- | --- |
| mode=direct | lat | -90..90 | geographic latitude (DegDec) |
|  | long | -90..90 | geographic longitude (DegDec) |
|  | elev | [number] | elevation of a geographic location |
|  | fov | 0..100 | [optional] field of view (%) to be occupied by target. Defaults to 50% |
|  | slew | [csv] | [optional] comma-separated list of camera OBJIDs. Those cameras will be repositioned |

Example:

target=target_123&mode=direct&lat=38.91145&long=-77.47006&elev=5&slew=101,102