PTZ Front-end Components Requirements

Goal: we need reliable, highly-customizable/configurable front-end components for Camera Pan-Tilt-Zoom (PTZ) commands executed from both On-Screen (Virtual Controller) and Hardware Joystick

Deliverables:

- On-screen XML-configurable “Virtual PTZ Controller”

- Behind-the-scene XML-configurable component for hardware joystick support

Specific Requirements:

- Generic requirements:

  - Mandatory: components should be embed-able into HTML pages

  - Optional/desired: components are to be usable from .Net framework

- “Virtual PTZ Controller” requirements:

  - Adobe Flash technology to be utilized for cross-platform portability

  - Set of call-backs to be provided for tight integration into browser pages

  - Component visual appearance should be fully-configurable (see specifics below)

  - Component should implement control primitives listed below

- “Hardware Joystick Component” requirements:

  - Hardware Abstration/Mappings to be provided (see below)

  - Implementation should provide abstraction layer to allow for cross-platform (Windows/Mac) and cross-browser (IE/Firefox) capabilities

  - Initial implementation may be limited to partial list of platforms/browsers supported

  - Java technology is recommended but not a strict requirement as long as cross-platform support is provided

Virtual PTZ Controller specifics

Proposed “Virtual PTZ Controller” (VPC) functionality is massively based on successful implementation which has been utilized in SKM since Release 2.3 and which source code may be found in next location: http://svn.dev.videonext.net/cgi-bin/svn/va/branches/2.5.0/ptz/src/ (PTZ_tab.fla)

Existent component provides Flash-based container object, which is getting configured from the embedding HTML page through XML file to draw specified dimensions canvas and place “Virtual Controls” into it based on that same configuration XML. Basically, it allows to specify any layout imaginable utilizing primitives like Joystick/Lever/Button/Text/etc..

Limitations of existent implementation are: insufficient number of controls, partial support for configurable look&feel, absence of camera feedback visualization.

The scope of this effort is defined in the hierarchical list below:

- REQUIREMENT: Fully customize look&feel of the component on the screen

  - Component window settings

    - Width and Height

    - Container type: fixed or resizable (may be mixed if Containers are used - see below)

    - Controls allocation: fixed or automatic layout-based

  - Component colors and background

    - Colors are to be applied to all internal objects

    - Background to be provided as either programmable gradient tone (configured through colors) OR image file (GIF/JPEG)

- REQUIREMENT: Provide more controls and extend their functionality

  - Combined Pan-Tilt-Zoom Joystick component

    - Single-click and drag provides immediate control over Pan-Tilt

    - Double-click enables/disables “sticky” mode, where Pan-Tilt is controlled by mouse positional movement and Zoom is controlled by mouse wheel

    - Component is NOT customizable in mapping into API commands, but rather hardcoded to SKM PTZ API command set

    - Control sends commands as soon as Operator changes Joystick deflection, but no more than 3 commands per second. No commands sent if no change in deflection

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label

  - Lever component

    - Control sends commands as soon as Operator changes Joystick deflection, but no more than 3 commands per second. No commands sent if no change in deflection

    - Two modes of operation:

      - Immediate control with single-click&drag

      - “Sticky mode” with double-click: Operator double-clicks to activate mode and mouse pointer “sticks” to lever’s head, so until another double-click Operator controls parameter moving mouse

    - Configurable parameters

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label

      - Direction (horizontal/vertical)

      - PTZ API command mapping

  - Cone/Pie-chart component

    - It is dual-purpose component: it shows current parameter values in the range and allows to control it directly. Primary use cases: Pan/Zoom, Tilt/Zoom

    - Component should visualize parameter change made externally (by another operator or by another control)

    - Component should allow control for positional and angle parameters (see Picture 1, marks 1&2) with click-drag-release on either cone/centerline or on cone external lines

    - Component should visualize both parameters boundaries by shading/hair-lines (see  Picture 1, marks 3&4)

    - Visually, is should be one of next shapes:

      - Full circle

      - 180-degrees fragment: top/bottom/left/right

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label

      - Shape type

      - Value limits for position and cone (counting zenith/vertical as “zero” for position)

      - Legend letters (if any): N/S/W/E, etc..

      - PTZ API commands mappings for up two control commands and up two notifications about parameters change

  - Pie-chart component

    - This is single-parameter control component with both control and parameter visualization

    - Functionality is very similar to “Cone/Pie-chart” described above with exception it works with one parameter contorl/visualization only

  - Slider control

    - This is another control/visualize component which allows to show current parameter value over the linear scale and change it

    - Component should allow receiving notifications about parameter change which may happened from another operator and/or control

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label

      - Direction: vertical/horizontal

      - Parameter range

      - Control sensitivity: linear/exponent/logarithmic

      - PTZ API commands mappings for control command and notifications about parameter change

  - Button control

    - Universal component providing both control and status visualization (optional)

    - Status visualization is provided by letting button to be lowered/raised to indicate status

    - Component may be embedded either standalone or within Group (see item 2.h below)

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label and placement (on button/left/right/top/bottom)

      - PTZ API commands mappings for control command and notifications about parameter change

  - Check-box control

    - Universal component providing both control and status visualization (optional)

    - Status visualization is provided by letting button to be lowered/raised to indicate status

    - Component may be embedded either standalone or within Group (see item 2.h below)

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label and placement (left/right)

      - PTZ API commands mappings for control command and notifications about parameter change

  - Group

    - Logical concept for combining several Buttons or Check-boxes together to create “one-of-many” behavior

    - Automatically sends commands through controls as needed to ensure only one of controls is enabled

  - Container

    - Concept for placement multiple components into box which is utilized by Layout Manager to rearrange components as needed if Auto-Layout is selected

    - Container is an OPTIONAL entity and may be skipped for Fixed Layout

  - VPC Tab

    - Optional entity that allows to sub-divide control real estate into tab-accessible sections

    - If feature utilized - configuration XML file should have separate canvas/layout for each tab

  - Selection-box control

    - Universal component providing both control and status visualization (optional)

    - Visually control is a combo-box-style with drop-down list opening upon click on correspondent icon

    - Status visualization is provided by showing text of selected entry

    - Mapping should be provided between entry visualized and value sent to API

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label and placement (on button/left/right/top/bottom)

      - PTZ API commands mappings for control command and notifications about parameter change

  - Text-Label

    - This is not a control, but text constant facilitating in providing the user experience

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Text label

  - Box

    - This is not a control, but visual element facilitating in providing the user experience

    - It is used to create box / line

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

  - Display component

    - This is not a control, but visual element facilitating in providing the user experience and reflecting realtime state/status information

    - Visually it is a text area controlled from web page through callbacks

    - Configurable parameters:

      - Dimensions

      - Screen position (for Fixed layout)

      - Background/font properties and initial value

      - Mapping to receive notifications

- REQUIREMENT: Visualize camera/system feedback/status information

  - This requirement is grossly covered in requiring specific components to work with current camera parameters via query/notifications (above)

  - If query/notifications are not available - it is expected that external container will utilize “Display component” to show “Camera Status not available” message

Hardware Joystick support specifics

Concept definition: If operator station is equipped with hardware joystick of some sort and it is supported by OS - it should be possible to map subset of VPC functions to hardware joystick. Per-user granularity in mapping definition is expected.

Since this document is concerned mostly of component functionality, subset of requirements relevant to component implementation will be explained. The integration is left out of this document scope.

The scope of “Hardware Joystick support” effort is defined in hierarchical list below:

- Component[s] provided should be suitable for embedment into HTML page

- Capability to query and adjust to specific hardware properties should be provided

  - Number of axis

  - Number of buttons

- XML-file-based configuration should be implemented to map each of available axis' and buttons to PTZ commands on VPC which is expected to be present on the screen at the same time

  - Axis mappings and directions

  - Button mappings

- Component should be cross-platform with support of Windows and Macintosh. Linux support is desired but optional at the moment. One of potential ideas on implementation: http://sourceforge.net/projects/javajoystick/